Practical No:-5

Implement a C program to perform division of two binary numbers (unsigned) using restoring division algorithm.

#include<stdio.h>

#include<conio.h>

#include<math.h>

int a=0,b=0,c=0,com[5]={1,0,0,0,0},s=0;

int anum[5]={0},anumcp[5]={0},bnum[5]={0};

int acomp[5]={0},bcomp[5]={0},rem[5]={0},quo[5]={0},res[5]={0};

void binary(){

int r,r2,i,temp;

a=fabs(a);

b=fabs(b);

for(i=0;i<5;i++){

r=a%2;

a=a/2;

r2=b%2;

b=b/2;

anum[i]=r;

anumcp[i]=r;

bnum[i]=r2;

if(r2==0){

bcomp[i]=1;

}

if(r==0){

acomp[i]=1;

}

}

c=0;

for(i=0;i<5;i++){

res[i]=com[i]+bcomp[i]+c;

if(res[i]>=2){

c=1;

}

else

c=0;

res[i]=res[i]%2;

}

for(i=4;i>=0;i--){

bcomp[i]=res[i];

}

}

void add(int num[]){

int i;

c=0;

for(i=0;i<5;i++){

res[i]=rem[i]+num[i]+c;

if(res[i]>=2){

c=1;

}

else

{

c=0;

}

res[i]=res[i]%2;

}

for(i=4;i>=0;i--){

rem[i]=res[i];

}

}

void shl(){

int i;

for(i=4;i>0;i--){

rem[i]=rem[i-1];

}

rem[0]=anumcp[4];

for(i=4;i>0;i--){

anumcp[i]=anumcp[i-1];

}

anumcp[0]=0;

}

void main(){

int i;

clrscr();

printf("\t\tRESTORING DIVISION ALGORITHM");

printf("\nEnter two numbers to division:");

printf("\nBoth must be less than 16");

do{

printf("\nEnter A:");

scanf("%d",&a);

printf("Enter B:");

scanf("%d",&b);

}while(a>=16||b>=16);

printf("\nExpected Quotient=%d",a/b);

printf("\nExpected Remainder=%d",a%b);

if(a*b<0){

s=1;

}

binary();

shl();

for(i=0;i<5;i++){

add(bcomp);

if(rem[4]==1){

anumcp[0]=0;

add(bnum);

}

else{

anumcp[0]=1;

}

if(i<4)

shl();

}

printf("\n-------------------------");

printf("\nsign of the result=%d",s);

printf("\nRemainder is=");

for(i=4;i>=0;i--){

printf("%d",rem[i]);

}

printf("\nQuotient is=");

for(i=4;i>=0;i--){

printf("%d",anumcp[i]);

}

getch();

}

Output :

RESTORING DIVISION ALGORITHM

Enter two numbers to division:

Both must be less than 16

Enter A:4

Enter B:2

Expected Quotient=2

Expected Remainder=0

-------------------------

sign of the result=0

Remainder is=00000

Quotient is=00010